Manual RGB LED Color Mixing

Gusti S. Halima

ENGR 102 Intro to Electronics

Shoreline Community College

October 2, 2017

Description of the problem and reason for building the circuit

This circuit is based from the 3rd circuit of SparkFun’s SIK Guide. I chose this circuit because it would be interesting to see how the basic colors of an RGB LED will look like by manually mixing the colors.

Things to do with the circuit

There are three buttons on the circuit which individually represent the colors red, green, and blue from the RGB led. Pressing a single button will light a specific color but it is also possible to mix between two colors or all at once.

Prior knowledge needed to create the circuit

A basic understanding to create a circuit and write code for individual LEDs and assigning them to their own buttons are needed. It is also recommended to understand how different resistors will affect the output of a color from the RGB LED.

Parts Required:

- Arduino Board

- Breadboard

- 6 long jumper wires

- 4 small jumper wires

- 3 buttons

- 1 RGB LED

- 2 1kΩ resistor

- 1 330Ω resistor

- 3 10kΩ resistor

Problem-Solving Approach

This circuit was simple to design which required no complicated planning. I created the circuit first then wrote the code.

Problems:

I had initially used the same 330Ω resistors for all three colors of the RGB led which resulted significantly brighter blue and green LEDs compared to the red LED. This problem was solved by changing the blue and green resistors to a 1kΩ resistor. However, it is also possible to use the same resistors for all three LEDs by setting the range values according to the RGB LED’s value spectrum which can be found online.

Schematic

Figure 1. schematic of the circuit

Figure 2. PCB of the circuit

Printed Circuit Board

Output Examples

Figure 3. RGB LED turns off by default

Figure 4. pressing the top button lights a red LED

Figure 5. pressing the middle button lights the green LED

Figure 6. pressing the bottom button lights a blue LED

Figure 7. example of mixing two colors; red and green LEDs

Figure 8. result of pressing all buttons simultaneously

Code

- /*

- * -Name: Gusti S. Halima

- * -class: ENGR 102, Fall 2017

- * -Project #1: Manual RGB LED Color Mixing

- * -What it does: The RGB LED main color LEDs (red, green, blue) are seperated

- * by individual buttons. Pressing the buttons simultaneously will mix the colors

- */

- //Creating arrays to store buttonPins and led pin values

- int buttonPins[] = {11, 12, 13}; //blue, green, red

- int led[] ={3, 4, 5}; //blue, green, red

- //variables to check the state of the buttons assigned for each led

- int redState, greenState, blueState;

- void setup() {

- //individually assigned pins

- pinMode(buttonPins[0], INPUT);

- pinMode(buttonPins[1], INPUT);

- pinMode(buttonPins[2], INPUT);

- pinMode(led[0], OUTPUT);

- pinMode(led[1], OUTPUT);

- pinMode(led[2], OUTPUT);

- /*

- * Creating a loop to assign the LED outputs

- * resulted a dim red LED, problem solved by

- * individually assigning the LEDs

- */

- }

- void loop() {

- blueState = digitalRead(buttonPins[0]);

- greenState = digitalRead(buttonPins[1]);

- redState = digitalRead(buttonPins[2]);

- /*

- * note that when a button's state is HIGH / 1 it actually turns

- * off the LED (buttons are inverted by default, search web for more

- * info). Either remeber this or change the buttons' pinMode from

- * output to INPUT_PULLUP will turn LED on if buttons are HIGH / 1.

- */

- //if-else statement for default colors

- if ((blueState == 0) && (greenState == 1) && (redState == 1)) {

- lightLED(HIGH, LOW, LOW);

- }

- else if ((blueState == 1) && (greenState == 0) && (redState == 1)) {

- lightLED(LOW, HIGH, LOW);

- }

- else if ((blueState == 1) && (greenState == 1) && (redState == 0)) {

- lightLED(LOW, LOW, HIGH);

- }

- //if- else conditionals to mix the colors

- /*

- * Initially couldn't mix the colors inside the RGB LED from using a single 220 ohm

- * resistor connected to the GND. Fixed by giving each LED their own reistor.

- */

- else if (blueState == 0 && redState == 0 && greenState == 1) { //red + blue

- lightLED(1, 0, 1);

- }

- else if (blueState == 1 && redState == 0 && greenState == 0) { //red + green

- lightLED(0, 1, 1);

- }

- else if (blueState == 0 && redState == 1 && greenState == 0) { //green + blue

- lightLED(1, 0, 1);

- }

- else if (blueState == 0 && redState == 0 && greenState == 0) { //all

- lightLED(1, 1, 1);

- }

- //When all buttons are high (not pressed), the RGB LED is turned off by default

- else {

- lightLED(0, 0, 0);

- }

- }

- /*

- the following method reduces the redundancy

- of typing digitalWrite() for each combinations.

- */

- void lightLED(int b, int g, int r)

- digitalWrite(led[0], b);

- digitalWrite(led[1], g);

- digitalWrite(led[2], r);

- }

Video: https://www.youtube.com/watch?v=Sz1xbOAvX4w

Resource: https://cdn.sparkfun.com/datasheets/Kits/SFE03-0012-SIK.Guide-300dpi-01.pdf

Syntax Highlighter used: http://www.planetb.ca/syntax-highlight-word

Schematic created using Fritzing